Second District Heating Efficiency Improvement Project (DHEIP2)

Second District Heating Efficiency Improvement Project (DHEIP2)

TERMS OF REFERENCE

CONSULTING SERVICES FOR TECHNICAL SUPERVISION AND PROVIDING SUPPORT IN BIDS EVALUATION FOR THE CONSTRUCTION OF COGENERATION PLANTS BASED ON GAS ENGINES AND HIGH-VOTLAGE POWER FACILITIES AT THE COMBINED HEAT AND POWER PLANT SOURCE-3 AND THE HEAT ONLY BOILER WEST

- PROJECT BACKGROUND

District heating (DH) is the dominant form of residential space heating in Chisinau, Moldova, where about 500,000 people rely on this type of heating for their well-being. Besides, the DH system supplies heat and domestic hot water (DHW) to public buildings and institutions, social facilities such as schools, hospitals and kindergartens, and to a number of commercial and private entities. The heat generated by the DH system is used both for space heating in the buildings and for supplying DHW. JSC Termoelectrica (TE) is the main heat generation and supply company at regulated prices. TE also generates electricity at its combined-heat-and power plants (CHPs), which is supplied to the national power greed at regulated prices.

Since 2008, Government of Republic of Moldova has engaged in an institutional, corporate and financial restructuring of Chisinau DH sector. Following several analytical studies and successful reform actions, the World Bank (WB) approved in 2014 the District Heating Efficiency Improvement Project (DHEIP/P132443) in the amount of US$ 40.5 million with the development objective of contributing to improved operational efficiency and financial viability of the new district heating company (TE) and improving quality and reliability of heating services delivered to the population in Chisinau.

The DHEIP was successfully implemented, supporting priority investments aimed at optimization and modernization of the heating distribution network, such as: modernization of the three main pumping stations, construction of a new pumping station, replacement of the priority DH networks, replacement of the Central Heating Substations (CHS) with automated Individual building level Heating Substations (IHS) in the residential buildings, technological upgrade of pumping systems at CHP-2 and Heat Only Boiler (HOB) West.

The Government of the Republic of Moldova has addressed a request to the WB to support further investments through the Second District Heating Efficiency Improvement Project (DHEIP2 or the Project).

DHEIP2 is based on the successful implementation of DHEIP. As DHEIP was completed on June 30, 2020, the DHEIP2 will finance activities to increase the efficiency of the DH system in order to continue the support for the improvement of the quality of DH and Domestic Hot Water (DHW) supply services in Chisinau.

DHEIP2 will finance the following investments:

Component 1. Optimization of Heat and Electricity Generation:

  - Modernization of Generation at CHP Source-1 through:

- reconstruction of Unit 2, including, inter alia: i) reconstruction of steam turbine high-pressure equipment; ii) retrofit of low-pressure equipment; iii) power generator diagnosis; iv) retrofit and modernization of vibration control module; v) replacement of turbine supporting equipment; vi) replacement of steam boiler burners and other equipment to increase boiler’s efficiency; and vii) installation of automated control module for capacity regulation and burning;

- major overhaul of turbine and replacement of boiler heat surface at Unit 3, inter alia: major overhaul of steam turbine high-pressure equipment; and, replacement of steam boiler’ economizer.

  - Increasing Efficient Cogeneration through:

- installation of efficient cogeneration modular units (gas engines) at HOB West and CHP Source-3; and

- installation of power transformers and switch gears, and any other equipment for connection to the power grid at HOB West and CHP Source-3; and

1.3. Development and installation of a management information system for Termoelectrica.

1.4. Development and installation of a Hydraulic Simulation System.

Component 2. Energy Efficiency Investments:

Provision of energy efficiency investments in the public and residential buildings, including pilot investments, but not limited to:

- Installation of individual heat substations and associated piping in selected residential and public buildings;

- Pilot investments in reconstruction of internal heat and domestic hot water distribution systems in selected buildings by changing from vertical to horizontal distribution to allow heat consumption control at apartment level.

- Pilot investments for thermal rehabilitation of selected residential buildings with horizontal distribution systems.

The Project completion is intended for December 31, 2025.

Moldova Energy Projects Implementation Unit (MEPIU) under the Ministry of Economy and Infrastructure of the Republic of Moldova is the entity responsible for the implementation of the Project.

In order to achieve the Project objectives MEPIU (“the Client”) will engage a suitably qualified Construction Supervision Consultant for subcomponent 1.2. (“the Project Manager” or “the Consultant”) to provide support in Bids Evaluation, Contract Administration and Supervision services for the construction of cogeneration plants based on Gas Internal Combustion Engines and high-voltage power facilities at the CHP Source-3 and the HOB West (Design Supply and Installation type of contract).

The Services to be provided by the Consultant will be divided into three (3) Phases that will last about 35 months. The first Phase (I) will include 2 months for bids evaluation support, the second Phase (II) will include 1 month for pre-Commencement activities, 20 months of works supervision, Completion and Operational Acceptance Certificates issue and other closing activities, and the third Phase (III) will cover the supervision and monitoring during the 12 months of Defect Liability Period (immediately after operational acceptance of the works) and the services will be contracted on a basis of a time-based remuneration. This Consulting Services Contract shall include, but not be limited to (a) bids evaluation support, contracting support and establishment of systems for contract administration and site supervision for the Design, Supply and Installation Contract; (b) support for the administration of the Contract; (c) control quantities and Contract costs; (d) monitoring and reporting the progress of the activities and maintaining technical records; (e) verification and certification of Contractor’s interim and final payment certificates; (f) the acceptance and/or approval of Contractor’s key staff, insurances, guarantees, licenses, programs, method statements, traffic management plans, safety measures (including COVID-19 safety measures plan), labour management procedures, suppliers and materials for incorporation in the activities, the quality assurance and control plans, laboratory provisions and execution of the testing program (if needed), subcontractors, equipment and environmental protection; (g) direct supervision of the activities and monitoring of progress including weekly analysis of the facilities installed against the approved plan by the Beneficiary/the Employer and reporting outcomes to the Client; (h) preparation of progress, technical and contractual reports; (i) monitor execution of the control tests of all materials; (j) holding bi-weekly progress meeting with the Contractor/s, recording and submitting signed copies of minutes to the Client; (k) supervision of testing activities, including the guarantee tests; (l) supervision and monitoring during the Defect Liability Period.

The Consultant shall prepare technical and financial proposals for all Phases (I, II and III) and will be evaluated for the aggregate price.

# PROJECT ORGANISATION AND MANAGEMENT

The Project will be managed on a day-to-day basis by the MEPIU/Client on behalf of JSC Termoelectrica (the Beneficiary). To support the Client in all aspects of bids evaluation, draft contract and support during contract signing, Plant Design, Supply and Installation contract implementation, including administration and supervision of the contract’s implementation, including environmental and social safeguards, the MEPIU intends to select a firm or consortium of firms (the “Consultant”) to provide support in bids evaluation and supervision (contract management) services to the Client.

- OBJECTIVES

The overall objective of the assignment is to act as the Supervision Consultant (“the Consultant”) for the contract that covers the scope of subcomponent 1.2 of DHEIP2 Project, namely towards Installation of efficient cogeneration plants based on Gas Engines at HOB West and CHP Source-3 and Installation of power transformers and power facilities/switchgear” (Plant Design, Supply & Installation).

In doing so, the Consultant will have the role and attributions as defined in these ToR. In addition, the Consultant will assist the Client with all aspects concerning bids evaluation, contracts administration, review all costs and contractors’ invoices, and environmental and social (ES) requirements specific to the Supervision Contracts.

With the involvement of the Consultant, the Client will have access to the best practice in the implementation of the Project administration of Design, Supply and Installation Contracts, as well as advice on specific technical issues including ES requirements.

The main objective of the assignment for the Consultant is to provide support in bids evaluation, to elaborate and issue the Bids Evaluation Report and to assist the Client to ensure technical supervision of the Plant Design, Supply and Installation Contract for the subcomponent 1.2. - construction of cogeneration plants based on Gas Internal Combustion Engines (GICE) and high-voltage power facilities at the CHP Source-3 and the HOB West with due diligence, to carry out the duties assigned to him in the Contract and provide other services as described in these ToR.

The Facilities that will be constructed/installed under the Plant Design, Supply and Installation Contract will include two new GICE-based cogeneration plants (CHPs) with a total net heat production capacity of at last 43 Gcal/h (total capacity for both plants) and a corresponding electricity production capacity, including all the necessary constructions, equipment and systems (GICE units and associated equipment, natural gas systems, electrical systems, control systems, flue gas systems, district heating (DH) pumping systems, etc., connections to DH system, power grid, natural gas network, water supply & sewerage, etc.), medium voltage (6 kV) and high voltage (110 kV) power facilities, including switchgear and power transformers, and associated equipment/systems.

The Facilities will be installed at two different locations/sites located in the city of Chisinau – at CHP Source-3 site, and at HOB West site.

- SCOPE OF WORK AND REQUIRED OUTPUTS

The Consultant shall undertake the following tasks for each Phase:

PHASE I: BIDS EVALUTION SUPPORT

The Consultant shall facilitate a process whereby the goods, works and services for construction of cogeneration plants based on Gas Internal Combustion Engines and high-voltage power facilities at CHP Source-3 and HOB West are procured in a timely manner in accordance with the Bidding Documentation duly agreed by the World Bank, best international practice, technically sound and economically efficient procedures and in compliance with the WB Procurement Regulations.

The Consultant shall assist the Client in the organization and administration of the Bidding process, including:

- Support in the preliminary examination of the bids including verification, eligibility, bids security, completeness of bids, substantial responsiveness, and support in the detailed examination of the bids, including the use of Life – Cycle Costs in the evaluation criteria;

- Carry out a review of the environment and social parts of the bids in accordance with the requirements provided in the Bidding Document;

- The assessment on Bidders’ Financial Capacity and participation in the evaluation of financial offers. Carry out the complete technical evaluation of the received Bids in accordance with the qualification requirements and technical specifications provided in the Bidding Document;

- Support to the Client in preparation of clarifications requests on the technical and other aspects to received Bids;

- Preparation of the Bids Evaluation Report and recommendations for award, in collaboration with the Client;

- Support in liaison with the WB, as needed, to achieve the WB's ‘no objection’ to the Bids Evaluation Report and the recommendations for award;

- Participation and assistance in the resolution of any issues that may arise at any pre-contract clarification meetings (if required);

- Providing support to the Client in drafting the Contract for Plant Design, Supply and Installation and assistance during contract singing, drafting the minutes at contract signing; and,

- Assistance to the Client in addressing any procurement related complaints and in any debriefing meetings which may be required in accordance with the WB’s procurement regulations and guides.

PHASE II: SUPERVISION OF CONSTRUCTION WORKS

The Consultant shall assist the MEPIU and the Beneficiary in the management and administration of the Plant Design, Supply and Installation Contract on the technical supervision of the works for construction of cogeneration plants based on Gas Engines and high-voltage power facilities.

Tasks shall include, but shall not be limited to, the following:

- Direct supervision of the activities and monitoring of progress;

- Preparation of mandatory reporting;

- Provide contract management support to the Client as required, including contract supervision of the Site-Specific ESMP implementation;

- Keep track of the payments made by the Client to the Contractor, follow if payments that have been certified/approved are arranged and paid by the Client, if such payments are made in due time, and that appropriate control and record systems are in place towards the compliance with financiers and the reporting requirements, and otherwise notify the Client of any delay;

- Prepare an integrated time schedule for progress meetings with the various parties; attend meetings together with the Client to support the investment programme as a whole, seek response to reports, and discuss contract issues on a regular basis with the Client and other stakeholders; prepare and circulate minutes of the meetings, including follow-up actions required to ensure progress.

4.1. Supervision of the Plant Design, Supply and Installation Contract and monitoring of progress

- Commencement of Activities on Site

The Consultant will have the following obligations:

Within two weeks from the effective date of the Plant Design, Supply and Installation Contract the Consultant shall organise a meeting with the Client’s/Beneficiary’s and Contractor’s representatives and make a presentation regarding the main Contract’s provisions highlighting key responsibilities of all Parties and of the Consultant (not only technical but also contract administration matters shall be properly covered so that they are understood by all involved), establishing modus operandi and communication system and clarify the expectations regarding Contract’s execution including early warning systems in case any issues arise. The focus shall be made on collaboration between the parties and amicable settlement of issues.

Before the start of the construction/installation Activities on Site, the Consultant shall address, inter alia:

  - Asses and advise the Client in approving Contractor's insurance policies and guarantees;

  - Asses and accept the Contractor's Quality Assurance Plan in order to submit it to Beneficiary’s approval;

  - Facilitate any communication and attend any meeting between Contractor/s and the owners of other facilities (water, sewerage, telephone, electricity, gas), roads and railways administrators/owners sharing the road right-of-way; in particular, give advice on proposed modifications by the owners of facilities;

  - Ensure that Environmental, Health and Safety and Social provisions set out in the contract are respected;

  - Ensure that traffic operational safety is met before commencing the activities and issue any work plan or drawing in that respect;

  - Check correctness of co-ordinates and levels of all survey reference markers and require the Contractor to make an independent check; Check if the Contractor performs all the necessary geodesic and geological surveys at the construction sites;

  - Verify the equipment and materials proposed by the Contractor to be used for the construction/installation according to the Contractor’s Plant Design, Supply and Installation Contract (sources/manufacturers of equipment and materials, their types, specifications, parameters, capacities, materials, compliance to the requirements of the Bidding Documents / Employer’s Requirements, standards, norms, etc.; verify if not worse than accepted equipment/materials offered in the Bid), subsequently accept and advise the Client / Beneficiary on their acceptability/rejection;

  - Verify estimated quantities in the Price Schedules (PS) and promptly advise the Client of any prospective Time and Cost effects and make appropriate recommendations;

  - Review the detailed design solutions proposed by the Contractor and the quantities and specifications of the equipment and materials included into the design in order to ensure that all the necessary equipment and materials are included in order to construct/install the Facilities corresponding to the Bidding Documents / Employer’s Requirements (for the Plant Design, Supply and Installation Contract), to the installation requirements of equipment manufacturers, to the norms and legislation of the Republic of Moldova and best engineering practices;

  - Review the design documentation and drawings, provide comments to the Client/Beneficiary and Contractor. Verify if the Consultant’s comments and the Client’s/Beneficiary’s comments are taken into consideration in the design documentation and drawings. Verify if the Contractor has obtained all the necessary approvals and verification/expertise for the design documentation from relevant authorities.

The Consultant shall advise the Client/Beneficiary regarding the acceptability / shall recommend to Client’s/Beneficiary’s approval and shall endorse the design documentation and drawings. Upon commencement of the activities, in accordance with the terms of the contracts, execute and deliver Commencement Certificates and obtain the countersignatures of the Contractor.

  - If the Contractor does not receive possession of the site/s in accordance with the contract, or if a Contractor does not receive a Commencement Certificate within the period stipulated in the contract, assess whether the Contractor is likely to suffer delay and/or incur extra cost and notify the Client accordingly.

- Review and Monitor implementation of Environmental and Social Safeguards, Health & Safety provisions

According to the WB's environmental and social requirements, the Project is classified as a Moderate Risk Project.

During the Project preparation activities an appropriate environmental and social assessment (ESIA / ESMP) has been carried out in compliance with WB’s environmental and social safeguard requirements.

The planned construction works will take place on land plots and in buildings owned by TE and / or the Government of the Republic of Moldova.

Therefore, a low environmental and social impact is expected such as: waste generation, noise, dust, and occupational safety during construction. There will be no economic or physical displacement and involuntary resettlement.

The Project's activities will not generate negative environmental and social impacts. However, in order to address the potential risks and temporary impacts caused by the construction works, the appropriate plans for mitigating environmental and social risks and impacts have been developed in accordance with WB requirements and the national legislation (Environmental and Social Management Plan (ESMP); Stakeholder Engagement Plan (SEP); Labor Management Procedure (LMP)).

The Consultant’s responsibility will include monitoring the implementation of the environmental and social mitigation measures during construction works and after the works are completed in order to ascertain that the mitigation measures are implemented in full compliance with the ESMP requirements and Specifications.

The Contractor shall develop its Contractor’s Environmental and Social Management Plan (“C-ESMP”) and Contractor’s Labor Management Procedure (C-LMP).

Prior to the commencement of construction works, the Consultant shall review and recommend for approval to the Client the C-ESMP and C-LMP developed by the Contractor.

The Consultant will ensure that all applicable environmental and social safeguards requirements of the Bank are being adhered to by the Contractor prior to construction works commencement. This includes:

- Review, recommend to Client’s approval and endorse the Environment and Social Management Plans developed by Contractor, including all updates at frequencies specified in the respective contract/s and Moldovan legislation;

- Review, recommend to Client’s approval and endorse the Contractor’s safeguards documents such as C-ESMP, C-LMP, Code of Conduct and other documents as per requirements of Bidding Document; which will cover, but not limited to, occupational health and safety, sexual exploitation and abuse (SEA) and gender-based violence (GBV), community Health and Safety, construction impacts management and other as per requirements of Environmental and Social Standards (ESSs) of World Bank;

- Review and consider the ES risks and impacts of any design change proposals and advise if there are implications for compliance with the applicable environment and social laws of the Republic of Moldova and safeguards requirements of the World Bank, the Environmental Permit and other relevant Project documents/requirements;

- Undertake audits, supervisions and/or inspections of the construction sites where the Contractor/s is undertaking activities under the contract, to verify the Contractor’s compliance with the Site-Specific ESMP requirements (including compliance with the applicable laws of the Republic of Moldova and other obligations);

- Undertake audits and inspections of Contractor’s accident and grievances logs, community liaison records, monitoring findings and other ES and H&S related documentation, as necessary, to confirm the Contractor’s compliance with ES/H&S requirements and Code of Conduct;

- Ensure, that all potential project affected parts, including communities living within the boundaries and surrounding areas are properly informed and provided with information of the project activities, environment and social impacts, and contact information including GRM prior to any physical interventions and implementation of the construction activities;

- Undertake audits and work inspections carried out by the State Authorities in accordance with the Applicable Law and report result of audits and inspections to the Client;

- Supervise the Contractor to prepare and implement Contractor’s Health and Safety Plan and where necessary, require the Contractor to provide and maintain at their own cost all site organisation, accommodation for personnel, office, parking areas for vehicles, toilets, firefighting equipment and first aid kits, traffic signs, environmental and HS signs, provide security for the protection of the facilities or for the safety and convenience of the public or others by complying with applicable ES laws of Republic of Moldova;

- Advise the remedial action(s) and their timeframe for implementation in the event of a noncompliance with the Contractor’s ES obligations;

- Coordinate with all involved parties about the relevant meetings with stakeholders and other interested parties including site meetings, meetings with local communities and progress meetings to discuss and agree appropriate actions to ensure compliance with Site Specific ESMP;

- Follow if the Contractor’s actual reporting (content and timeliness) is in accordance with the Contractor contractual obligations and recommend any modifications if it is necessary;

- Monitor if relevant government’s policy commitments to gender equality are addressed in the Plant Design, Supply and Installation Contract no. 9134 – B1.2 “Installation of efficient cogeneration plants based on Gas Engines at HOB West and CHP Source-3 and Installation of power transformers and power facilities/switchgear” (Plant Design, Supply & Installation)”;

- Review, recommend and endorse, in a timely manner, the Contractor’s ESMP (including regular reports and incident reports) regarding the accuracy and efficacy of the ES documentation;

- Undertake liaison, from time to time and as necessary, with Project stakeholders to identify and discuss any actual or potential ES issues/risks and permits;

- Support the Contractor, Client and Beneficiary to the grievances redress mechanism (GRM) management (if required). Ensure, together with the Contractor, the resolution of all grievances received at site-level. Monitor and reporting on received grievances and status of their resolution;

- The Consultant will ensure that all applicable COVID-19 impact considerations, based on the Bank’s support guidance notes available at https://www.IBRD.com/sustainability-covid.html. are being adhered to and that the Client/Beneficiary is duly informed about Contractor’s compliance with the requirements.

The Consultant shall monitor and ensure that during the implementation of the Project, all employees, including the Consultant and the Contractor’s personnel, engaged in the activities related to the implementation of the Project, must fully respect prescribed measures for occupational protection and also additional measures related to COVID-19 pandemic, all in accordance with the recommendations of the World Health Organisation and relevant authorities of the Republic of Moldova.

- Design Review, Installation and Plant Supervision and Quality Control

- Draw up and maintain a schedule of all necessary statutory licences, permits and approvals necessary for the performance of the activities (the Schedule should identify the dates for submissions and approvals, and the person or persons responsible for making application for such licences, permits and approvals either on their own account or on behalf of the Client/Beneficiary or the Contractor). Check that such applications are made on time and assist in negotiations with other parties, as necessary from time to time;

- Coordinate with and assist the Contractor/Client to ensure that all permits required are obtained on time;

- Monitor validity of Contractor's insurance policies and guarantees and timely advise the Client on their expiry dates, necessity to request the extensions of the validity and where necessary change the amount of the insurance policies and guarantees;

- Provision and administration of the Project Management Information System (PMIS) for management of project correspondence and documents in accordance with the approved by the Client/Beneficiary PMIS plan and procedures, and timely updates of the records and reports thereof:

  - review and conclude for recommendation the Contractor’s site installation and supervise that the Contractor complies with all applicable legal requirements regarding site organisation;

  - review the proposed Contractor/’s equipment;

  - review and endorse the Contractor’s Traffic Management Plan (CTMP);

  - review and recommend for Client’s approval the procedures to ensure compliance with the Site Specific Environmental and Social Management Plan,

  - review and recommend for Client’s approval the Health and Safety procedures;

  - certification of measured quantities in accordance with the Plant Design, Supply and Installation Contract;

  - verify and recommend the earthmoving scheme for earthworks;

  - inspect all procedures for construction of drainage works (if needed);

  - inspect of setting-out of the facilities;

  - review of the Contractor’s documents and information management system;

  - comparison of activities output actually carried out with that presented in the Bid and discuss with the Contractor remedy to possible slippage;

  - inspect the facilities installation to check whether performance complies with specifications and drawings;

  - witness regular tests of materials and of facilities installed, and order additional tests if required;

  - order the removal of improper or substandard designed or installed facilities;

  - control materials incorporated into the facilities.

- Review Design documentation, including drawings submittals of the Contractor/s and recommend for its optimization. Check adequacy of the Contractor’s detailed design based upon own calculations and evaluations;

- Provide supervision over compliance by the Contractor with the requirements of organizational and technological documents that determine the technological process of construction and installation;

- Review the compliance of project documentation with the Client’s requirements, standards and local legislation and recommend for Client/Beneficiary’s approval the detailed design documentation before its transmittal for approval to relevant State Authority;

- Supervise the Design, Supply and Installation stages, assure technical assistance during design and facilities installation, inspect the materials, equipment and workmanship;

- Ensure supervision of Plant Design, Supply and Installation compliance with the requirements of the technical specifications;

- Supervise the Contractor’s individual tests of the equipment before and after installation, of facilities to be covered or put out of view and proper registration of their results;

- Carry out oversight inspection of the facilities being installed by the Contractor to provide assurance of the quality and standards of the materials and workmanship, and compliance with the specifications and drawings as included in the Contract, the Approved Design, the Detailed Drawings, the Method Statements, the Quality Assurance Manual and any agreed amendment thereto;

- Participate in the acceptance of equipment during the tests and inspections;

- Supervise and participate in the functional guarantee tests;

- Review and confirm tests and inspections reports, including the functional guarantee tests reports;

- Daily record site events including facilities installation location, activities and staffing on site, and quantities to pay (subject to quality tests) in a work site logbook;

- The Consultant and all supervisory / technical staff shall maintain a daily work diary recording site conditions, Contractor’s activities and incidents / events that may occur;

- Day-to-day measurement, verification and recording of quantities of works carried out by Contractor/s;

- Recapitulation of activities carried out monthly;

- Monthly comparison of actual progress against progress as scheduled;

- Review of Contractor’s Monthly Statements and Statement at Completion and verify and endorse Interim Payment Certificates;

- Review and endorse documents submitted for payment by the Contractor to confirm that the conditions for the respective payments are fulfilled according to the Contract;

- In case of documents for the supply of equipment and materials, the Consultant shall review, endorse and confirm that the respective equipment and/or materials correspond to the requirements of the Contract and are intended for the implementation of the Contract according to the approved design;

- Attendance at periodic site meetings and monthly progress meetings and ensuring minutes signed by all parties are recorded. Keep full and proper records of all meetings and discussions attended or conducted by the Consultant and make the records available for inspection by the Client/Beneficiary and the Bank forthwith on request;

- Notify the Client/Beneficiary immediately if the Contractor fails to complete any part of the facilities within the applicable Time for Completion or appears likely so to fail. If necessary, prepare for the Client/Beneficiary review a case for the application of liquidating damages for delays or a claim against the Performance Guarantee where a Contractor has failed to perform;

- Notify the Client on the need to postpone the activities within the Contract (with the obligatory subsequent written notification from the Client/Beneficiary and the Contractor) in cases of threat of deformation and collapse of structures, danger to life and health of people and the environment, as well as failure to comply with technical supervision instructions for the elimination of defects and violations;

- Financial management of the Plant Design, Supply and Installation Contract. Based on (i) Contractor's programme of activities and cash-flow predictions which should be revised at required time intervals and, (ii) upon his own judgement, the Consultant shall prepare, as part of its Monthly Reports, monthly disbursement tables showing the status of previous disbursements and a tentative prediction of future disbursements on a monthly basis;

- For the purpose of confirmation of the Contractor’s applications for VAT with the right of deduction, in accordance to Article 104 (c1) of the Fiscal Code of the Republic of Moldova, and exemption from payment of Customs, Import Duties and taxes for goods that, in the process of use, cause environmental pollution, in accordance with Government Decision No. 246 of April 8, 2010 (including further modifications/additions), the Consultant shall confirm to the Client that the Plant and Equipment imported for use under the contracts signed between the Contractor and different suppliers, are intended for the implementation of this Contract; The Consultant shall review and endorse the Contractor’s documents for import of equipment and materials to confirm that the respective equipment and/or materials correspond to the requirements of the Contract and are intended for the implementation of the Contract according to the approved design;

- Attend the inspections carried out on sites by the state authorities in accordance with the applicable law;

- Organise provisional technical acceptance of facilities and submit all supervision documents to the members of the Reception Committee according to the applicable law;

- Review and recommend the Operational Acceptance Certificate in accordance with the Conditions of the Plant Design, Supply and Installation Contract noting that no outstanding equipment or installations shall be left for Defect Liability Period (DLP) and the Consultant shall take this into account before issuing the Operational Acceptance Certificate as well as that the As-Built Drawings and manuals will have to be completed also in advance. Finally, the requirements of the applicable law on the works acceptance shall be taken into account by the Consultant as the compliance with these procedures is a precondition to the Operational Acceptance Certificate and thus advance notice will need to be given to the Client, so that the required Reception Committee can be timely formed; Calculate delay damages according to the Particular Conditions of the Contract and issue Notice of Client’s Claim for delay damages or any other claims, where applicable.

The required procedures and formats to carry out these tasks shall be prepared by the Consultant and submitted for the approval of the Client in the Inception Report.

The Consultant shall note that MEPIU/Employer/Client is under obligation to seek the Bank’s approval before agreeing to or implementing any modification or waiver of the terms and conditions of the Contract including granting an extension of the stipulated time for performance.

The Consultant will seek prior written approval of the Client for the following:

- verifying and endorsing any payment or Interim Payment Certificate;

- reviewing and recommending any changes in the Contract design;

- giving recommendations for issuing of any Change Order.

- in the event of additional activities, the Consultant shall report on the relative merits of tendering vis-a-vis issuing a Change Order for such additional activities;

- reviewing of Price Adjustment Indices and base values;

- making appropriate recommendations towards proposal(s) for Change submitted by the Contractor/s;

- determining any new rate or price with respect to any Change;

- recommending (after Bank’s No-objection) any extension of the Time for Completion;

- granting any claim for any additional cost including any cost associated with extension of Time for Completion;

- suspending the performance of any or all Contractor’s obligations in accordance with GCC 41.

Any response by the Consultant which requires Client’s approval, except as otherwise expressly specified, shall be notified in writing to the Contractor within 30 days of receipt (14 days for the Consultant, 10 days for the Client, and 6 days for the Consultant to consider Client’s comments).

In case any delay in activities will be caused by slow response / initiative / determination or any other actions required and /or expected of the Consultant, the Consultant will have to then extend their services for the respective period without extra payment, unless the delay was outside his reasonable control and cannot be envisaged by the professional consultant experienced in the Services.

- Quality Control

- Develop a Quality Management Plan (QMP) for all aspects of the Project;

- Receive from the Contractor the full particularised version of their Quality Assurance Manual in English and local language; as soon as possible check and comment upon the same and, if necessary, request the Contractor/ to amend the same;

- Participate, document and ensure that Factory Acceptance Tests (FAT) and Site Acceptance Tests (SAT), covered by the Contractor, are executed according to the technical specifications for the cogeneration plants based on Gas Engines and high-voltage power facilities. The respective costs shall be covered by the Contractor. Issue and sign minutes on the compliance of the tests with the technical requirements of the Contract;

- Monitor and participate in all commissioning, testing and acceptance procedures, as well as other testing procedures required by Client;

- Inspect/review the Contractor’s and Sub-Contractors testing procedures;

- Notify the Client if there is any failure of tests or inspection and if such failure is anticipated to cause delay to any Completion Date or other material adverse consequence; advise on further tests required and arrange that the Contractor carry out necessary rectification;

- Carry out oversight inspection of the work being executed by the Contractor to provide assurance as to the quality and standards of the materials and workmanship, and compliance with the specifications and drawings as included in the Contract, the Approved Design, the Detailed Drawings, the Method Statements, the Quality Assurance Manual and any agreed amendment thereto;

- Request the Contractor to make available for review copies of all test results within a reasonable time of the test being carried out;

- Construction Risk Assessment

The Consultant will have the following obligations:

- Identify the hazards for the construction stage. This must comprise procedural risks, environmental and social risks, technical and quality risks, risk of cost overrun, risk of delays, risk to third parties, etc.

- Assess the likelihood of these risks and the potential consequences;

- Identify possible mitigation measures; and

- Proactively and continuously seek to manage and reduce/eliminate hazards/risks.

- Archaeological Remains

The Consultant will have the following obligations:

- Receive, analyse and recommend the procedure of archaeological remains and find chance procedure developed by the Contractor. The procedure shall establish, inter alia, the communication channel to the Consultant and State Archaeology Authority in the event of the discovery of any fossils, coins, articles of value or antiquity or other similar remains, dangerous dumpsites, hazardous contamination or munitions within the Site and advise the Client in relation to the steps to be taken in consequence thereof and the time and cost implications of such steps;

- Decide any extension of time and any additional payment to which the Contractor is entitled in accordance with the terms of the Contract as a result of discovery of any archaeological remains or dangerous dumpsites;

- Where necessary, supervise the Contractor to provide and maintain at their own cost all lights, guards, fencing, warning signs and watching, for the protection of the facilities or for the safety and convenience of the public or others.

- As-Built Drawings and Documentation

The Consultant will have the following obligation:

- Supervise, verify and indicate the appropriate comments/recommendations regarding the Contractor’s “as-built” drawings.

- Site Management and Monitoring Progress

The Consultant will have the following obligations:

- Attend site meetings and endeavour to ensure that at all times outstanding problems are settled in order to avoid any delay or extra expenditure (having regard, however, to the terms of the Contract and the limitations on the authority of the Consultant referred to under these ToR).

- Keep full and proper records of all meetings and discussions attended or conducted by the Consultant and make the same available for inspection by the Client forthwith on request.

- On request of the Client, attend meetings to resolve differences of opinion on general or technical matters.

- Ensure that the Client receives timely notice of and is permitted to attend all site meetings and other meetings with the Contractor/s.

- Notify the Contractor if the Consultant objects to any person who has conducted himself as incompetent or negligent; notify and advise the Client if this is the case and agree on any action to be taken.

- Advise the Client on the general organisation of the Contractor’s resources at the Site, including management and programming systems, manpower, plant and equipment.

- Advise the Client on compliance by the Contractor with respect to sub-contracting (if the case), as specified in the Plant Design, Supply and Installation type of Contract.

- Report on the progress of the activities to ensure completion within the time established in accordance with the Contract;

- If for any cause other than those listed in the Contract, the rate of progress of the activities or any part of the facility is at any time, in the Consultant’s opinion, too slow to ensure the completion of the activities or any part of the facility by the Completion Date, instruct the Contractor in accordance with the conditions of Contract in writing with a copy to the Client;

- Receive from the Contractor due copies of formal quarterly and monthly Progress Reports, in accordance with the Contract/s, checking the same to ensure that they cover all relevant aspects of the Design, Supply and Installation Works, and highlight actual or potential departures from the Programme or the Milestone, Payment or Design Submission Schedules and stating the proposed or necessary measures to be taken by the Contractor to overcome such departures; commenting on and supplementing as necessary such Progress Reports before forwarding them to the Client, and advising of any necessary measures to be taken to achieve completion of each part of the facility within the applicable Time for Completion;

- Convene formal monthly meetings (“Project Team Meetings”) and ad-hoc meetings, upon necessity, with Project stakeholders and other relevant parties. These meetings must have a formal agenda and minutes;

- Verify that the progress is in compliance with the Programme Schedule approved under the Contract/s on behalf of the Client. Notify the Client as far as possible in advance of any possible failure to attain the Design, Supply and Installation Programme by the applicable date or non-compliance with the Programme.

- Authorise, with Client’s prior approval, any reasonable request by the Contractor’s to work outside agreed time windows (e.g. at night or on locally recognised holidays) to expedite progress so as to comply with the Completion Date for the Design, Supply and Installation or any part of the facility.

- Check the provision of all necessary insurance, performance securities and warranties and other relevant contract documentation.

- Report immediately any environmental and social incidents and accidents occurred on construction site, indifferent of complexity and gravity and take actions by stopping construction activities and keep situations under control by communication with hospitals, firefighting authority and other state authorities.

- Ensure that occurred incidents/accidents are investigated properly and assist contractor to investigate incidents/accidents and report investigation results to the Client and State Authority for labour management.

- Default, Delays, Claims and Disputes resolution

- Check that the Contractor continue to give the details and notices that are required under the Contract relating to any delay, and forward such details and notices to the Client;

- Notify the Client immediately if the Contractor fails to complete any part of the facility within the applicable Time for Completion or appears likely so to fail;

- Assist the Client with analysis of circumstances and issue of recommendations when/if the Contractor/s considers himself to be entitled to any extension of the Time for Completion and/or any additional payment, under any Contract/s Clause or otherwise in connection with the Contract/s;

- Advise the Client on any difficulties that may arise generally in connection with the Design, Supply and Installation of the facilities;

- Assist the Client in the process of receiving and dealing with notice of the intention of a Contractor to claim any additional payment within the times stipulated in the conditions of Contract and adopt the stipulated process for claim resolution, notifying the Client thereof;

- Upon the request of the Contractor/Client, discuss the delay, the reasons therefore, determine and notify the Contractor of any extension of time and any amendments to any of the Milestone, Payment and Design Submission Schedules. Where the delay is the result of any of the causes referred to in the Contract or where an extension of time has been granted, consult with the Contractor on behalf of the Client, and send to the Client for its review, such revisions to the Programme, Milestone, Payment and Design Submission Schedules which the Contractor considers necessary in consequence of any such delay or extension of time;

- If any urgent remedial activities are necessary, act in accordance with the Contract, and otherwise advise the Client on carrying out the same by the Contractor or, if impossible to do so, discuss such failure with the Client.

- Work with the Client and the Contractor to set up the Dispute Board, as required under Conditions of Contract, and follow up all requisite processes in addressing the Contractor’s or Client’s claims.

- If necessary, prepare a case for the application of Liquidated damages for delays or a claim against the Performance Guarantee where the Contractor has failed to perform;

- If any dispute or difference is referred to arbitration, assist the Client generally in respect of such arbitration provided always that the Consultant will not be required to act improperly or contrary to his obligations under the Contract.

- In the event of termination, provide advice and assistance in connection with the departure of the Contractor from the site and the assignment of the benefit of any agreement for the Plant Design, Supply and Installation.

- Advise the Client of their rights upon the occurrence of any Force Majeure event.

- Completion, Commissioning and Operational Acceptance

- Assist the Client, within the terms and conditions stipulated in the General and Particular Conditions of Contract, with issues, upon receiving a request from the Contractor, to issue the Completion Certificate;

- After receipt of such a request, inspect the plant and facilities with a representative of the Client/Beneficiary within the time period specified in the Contract;

- Provide support to the Client in organising the Reception Committee;

- Taking into account any comments of the said parties, give instruction in writing to the Contractor specifying all the activities required to be executed by the Contractor before the issuance of a Completion Certificate; notify the Contractor of any defects in the Design, Supply or Installation of the facilities affecting Completion that may appear after giving such instructions and before completion of the activities specified therein; provided that the Contractor has completed the activities so specified and remedied any defects so notified to the satisfaction of the Consultant and the Client. Inform the Client about the possibility to issue a Completion Certificate within the period prescribed in the Contract/s;

- Upon acceptance of plant and facilities installed under the Contract/s, as well as in preparation for the commissioning of the facility, check the actual readiness of each type of structures, plant and facility as a whole, the availability of properly finalised as-build documentation and manuals, verify the availability of fixed and installed equipment listed in acts of acceptance, with actual availability on the site;

- Participate at the Functional Guarantee Tests of the Facilities to be performed by the Contractor/s intended to achieve the Functional Guarantees prescribed in the Contract and communicate to the Client/Employer of any deviations or the respective successful performance;

- Review and confirm functional guarantee tests reports; If applicable, calculate the amount(s) of liquidated damages for the failure to achieve functional guarantees according to the conditions of the Contract.

- Instruct the Contractor to search for defects and the cause thereof and to execute all such activities of amendment, reconstruction, and remedying defects, shrinkage or other faults during the Defect Liability Period as prescribed within the Contract;

- In the event that the Contractor refuses to carry out any rectification activities, assist the Client in proceeding to do such activities, following the issue of notice to the Contractor, the reasonable costs incurred by the Client in connection therewith to be paid to the Client by the Contractor or be deducted by the Client from any monies due the Contractor or assist the Client/Employer to claim them under the Performance Security;

- Advise the Client of the value of any completed part of the facilities and of any further information as may be necessary for calculating any adjustment in the amount of the Performance Security and any securities procured by the Contractor to secure its obligations;

- Notify the Client immediately if the Contractor is failing to comply with its obligations under the Contract. Discuss with the Client possible remedies, and advise on the rights and obligations of the parties under the Contract;

- As soon as may be practicable after any entry and termination by the Client/Employer in accordance with the Contract, assist the Client in adopting the procedures and conditions stipulated in the respective Contract;

- If any urgent remedial work is necessary, advise the Client on having it carried out by the Contractor or, if impossible to do so, discuss such failure with the Client;

- In the event of termination, provide advice and assistance in connection with the departure of the Contractor from the site and the assignment of the benefit of any agreement for the Plant Design, Supply and Installation activities;

- In the event that the Contractor refuses to carry out any rectification work, assist the Client in resolving the situation;

- Payments and Accounts

The Consultant should carry out its duties in relation to payment of the Contractor’s activities in accordance with the Contract/s:

- Receive from the Contractor on a monthly basis invoices pursuant to the Contract.

- Receive from the Contractor invoices for supply of equipment and materials when submitted by Contractor;

- Upon receipt of the Contractor’s invoices, copy to the Client each such invoice and give due consideration to any comments of the Client.

- Prior to certification of any payment to a Contractor/s in relation to a Contractor’s claim, the Consultant will have consulted with the Client on the grounds on which the Consultant intends to certify the payment. The Consultant will provide the Client with any particulars to enable the Client to establish its position with regard to the Consultant’s certificate.

- Check in accordance with the relevant provisions of the Contract the Contractor’s invoices and resolve with the Contractor/s, where possible, any mistakes and queries which may arise in conjunction therewith; advise the Client of any adjustments considered necessary.

- Ascertain the amount to be certified in respect of the Contractor’s invoices pursuant to the Contract. Within the times stipulated in the Conditions of Contract/s, issue a certificate addressed to the Contractor and the Client specifying the aggregate amount payable by the Client to the Contractor.

- Certify any additional amounts due to the Contractor in respect of valid claims notified in accordance with the procedure set out in the Contract.

- Consider any invoices submitted by the Contractor pursuant to the Contract and certify any additional sums which the Consultant is empowered to certify as due, provided always that the Contractor has supplied sufficient particulars to enable the Consultant to determine the amount due.

- Collate and prepare quarterly budgets showing sums anticipated to fall due from the Client to the Contractor, dates and amounts of invoices and certificates under the Contract and values expected to be achieved in the quarter and deliver the same to the Client forthwith.

- Fully comply with the procedure for disputed certificates set out in the Contract.

- Forthwith carry out any correction or modification of any certificate or other action authorised by the Contract whenever the Consultant considers it appropriate to do so.

- Within the times stipulated in the Conditions of Contract after receipt of this draft Final Payment Certificates (Completion and Operational Acceptance Certificates), work with the Contractor to agree a Final Statement.

- Maintain Contract accounts and prepare final Contract Statement.

- Change Orders within Plant Design Supply and Installation Contracts

The Consultant will note that the Client is under obligation to seek the Bank’s approval before agreeing to or implementing any material modification to the terms and conditions of any Contract.

The Consultant will assist the Client to comply with the procedures agreed with the World Bank.

Unless, in the opinion of the Consultant, an emergency occurs affecting the safety of life or of the Works or of adjoining property, the Consultant will provide a preliminary report to the Client on any prospective Change, outlining the basis for the Consultant’s valuation of the Change, including but not limited to the following:

- The Consultant’s opinion on the extent, if any, of applicability to the varied activities of the rates and prices set out in the Contract; when expressing an opinion, the Consultant will take into account the actual or expected currencies of cost (and the proportions thereof) of the inputs of the varied activities without regard to the proportions of various currencies set out in the Contract;

- The quantity and the value of the varied plant and/or facilities that can be determined using the rates and prices set out in the Contract;

- The quantity and the Consultant’s estimate of the value of the varied activities, which can be determined using the rates and prices set out in the contract as the basis for valuation: the Consultant will provide a detailed breakdown of the rates and prices set out in the Contract and identify the price components that the Consultant used or intends to use for the valuation of the varied activities;

- The quantity and the Consultant’s estimate of the value of the varied activities, which can only be determined using the rates and prices to be agreed upon between the Consultant and the Contractor. The Consultant will provide to the Client with an appropriate justification of the basis for the agreement.

In case of a Change, the Consultant will follow the procedures and conditions stipulated within the General and Particular Conditions of the Contract:

- However, in the event that the Client orders changes to the Approved Design, and if the compliance with such order has, in the opinion of the Consultant, materially delayed or is potentially likely to delay the Completion Date for the Supply and Installation of any facility, determine the extension of time and the additional payment to which the Contractor/s will be entitled in accordance with the terms of the Contract.

- Suspension

If in the Consultant’s opinion, a suspension of performance under the contract is required, the Consultant will initially consult with and seek the approval of the Client. After receiving their approval to issue a suspension, the Consultant will follow the procedures and conditions established in the Contract Conditions.

PHASE III: DEFECT LIABILITY PERIOD

During the third Phase (III) of the assignment which will cover the supervision and monitoring during the 12 months of Defect Liability Period (immediately after Operational Acceptance of the works), subject to the successful performance of the Consultant in Phase II, the Consultant shall provide sufficient on-site supervision of the Contractor’s operations during the whole Defect Liability Period of 12 months (if any), corresponding to the last section of the Project accepted as completed. The services will be contracted on a basis of a time-based remuneration.

During the entire Defect Liability Period, the Consultant will be responsible for supervision and monitoring the Plant and facilities. If during the Defect Liability Period any defect should be found in the design, engineering, materials and workmanship of the Plant supplied or of the work executed by the Contractor, the Consultant shall notify the Client/Beneficiary on identified deficiencies and recommend further actions to be done. The Contractor shall promptly, in consultation and agreement with the Consultant/ Beneficiary/Employer regarding appropriate remedying of the defects, and at its cost, repair, replace or otherwise make good as the Contractor shall determine at its discretion, such defect as well as any damage to the Facilities caused by such defect in accordance with the Clause 27 of the Plant Design, Supply and Installation Contract Conditions, no. 9134 – B1.2.  The Contractor shall provide to Employer/Beneficiary its written notice on remedying the identified defects and that the Facilities are fully in accordance with the requirements of the Contract and are ready for Consultant/Beneficiary testing/inspection. The Consultant shall inspect/test the remediated facilities and notify the Employer on the results.

4.2. Reporting Requirements and Time Schedule for Deliverables

During the period of the services, the Consultant will provide the Client with the following reports (2 copies in Romanian and 2 copies in English including their electronic versions in both languages):

Phase I:

- Bids Evaluation Report and recommendations for award: within 2 months of the effective date.

Phase II:

- Inception Report: within one month from the start of the Phase II;

- Consultant’s Services Program Report: within 21 days of receipt of the Contractor’s Work Program;

- Monthly Progress Reports: within 10 days of the end of each calendar month;

- Change Order Reports (if any): no later than 14 days from the moment when the Consultant becomes aware that a Contract Change order may be required;

- Report on Completion of Works: within 28 calendar days upon issuing the Completion Certificate for each the contract;

- Claims Reports (if required): a) for preliminary assessment of the validity of the Contractor’s potential Claim no later than 7 days from the date of receipt of the respective Notice to Claim; b) for a detailed analysis of the validity of the Claim no later than 21 days from the date of receipt of such Claim;

- Draft Final Report: Prior to last two months of the assignment;

- Final Report: immediately upon issuing the Final Payment Certificate (Operational Acceptance Certificates);

- Special Reports (if required): within two weeks from the Client’s request.

Phase III:

- Report on Defect Liability Period: no later than 14 days from completion of the 12 months of Defect Liability Period;

- Contract Completion Report (Phases I+II+III): no later than one month from the successful completion of the provision of all consulting services.

- Bids Evaluation Report

Within 2 months of the effective date the Consultant, in collaboration with the Client, shall submit the complete Bids Evaluation Report and recommendations for award. In case comments will be received from the World Bank, the Consultant shall provide support to the Client in revision/adjusting the Evaluation Report until “No-objection” is received.

- Inception Report

Within one month from the start of the Phase II, the Consultant shall prepare and submit to the Client the Inception Report showing Consultant’s models for the monthly reports, Interim Payment Certificates, refine staff mobilization schedule, time reporting systems, Project Management Information System, define the responsibilities to be delegated, establish the communication procedures with the Client, Beneficiary, Contractor and the required procedures and formats to carry out the Plant Design, Supply and Installation contract administration tasks.

- Consultant’s Services Program Report

Within 21 days of receipt of the Contractor’s Work Program, the Consultant shall submit to the Client the Consultant’s Services Program Report, which shall indicate the Consultant’s opinion and comments on the Contractor’s Work Program or any update and/or revision thereof.

In the case where the Consultant has made comments the Report will indicate the reasons for the comments.

In the event that the Consultant considers the Contractor’s Work Program acceptable, the Consultant’s Report will include the following:

- Copy of the Contractor’s detailed Programme of Performance, showing the order in which the Contractor intends to carry out the activities under the contract with the highlighted critical path/s, including general description of the methods which the Contractor intends to adopt, and of the major stages, in the execution and acceptance of the facilities, Contractor’s key equipment and personnel mobilization plan, and other information as may be deemed necessary and appropriate by the Consultant, including coordination arrangements with other Client’s, Beneficiary’s, Contractor’s personnel, if any;

- Copy of the Contractor’s detailed cash flow estimate, including a monthly breakdown of all prospective payments forecast to be made to the Contractor under the Contract;

    - A supporting report which will include:

      - A list of critical path items and the related Contractor’s estimates of delivery periods, accompanied by the Consultant’s estimate of the latest delivery periods for each critical path item to warrant the completion in accordance with the Contract;

      - Consultant’s opinion on the requirements in the Contractor’s proposed Program for the information the Contractor reasonably requires from the Client, reasonableness of the envisaged order and timing when that information is required, and all Client’s activities and constraints (such as approvals/reviews, etc.) and risks arising thereof;

      - For the manufacture of each main item of Plant and Materials, the name of the manufacturer, manufacture location and the expected dates of:

      - Commencement of manufacture;

      - Contractor’s and/or the Consultant’s inspections and tests; and

      - Shipment and arrival at the Site.

- Plant and materials selected by the Consultant for inspections and tests, including an appropriate specification of the tests to be carried out and the associated arrangements;

- Names and particularities, including gender, of the Contractor’s representative and other superintendence personnel recommended by the Consultant in accordance with the Contract;

- Names of subcontractors consented by the Consultant in accordance with the Contract and, for each subcontractor, the cost and quantity of the subcontracted Works; this section will include a brief justification for the Consultant’s consent;

- The details of a Project Management Information System (PMIS), acceptable to the Client, for efficient and timely management of correspondence and documents form the Contractor, Client and other stakeholder(s), as applicable, to be compatible with Client’s Management Information System (MIS) requirements, also including an implementation schedule.

      - Copies of all notices, consents, approvals, certificates or determinations given or issued by the Consultant within the reported period;

- Summary of the main actions with their estimated timing required of the Client during the Contract execution, summary of the issues under the contract guarantees and insurances with their key terms and matters for the client to pay specific attention to, such as expiry dates etc., and

- Other information as may be required by the Client.

- Monthly Progress Reports

The Consultant will prepare and submit to the Client Monthly Progress Reports within 10 days of the end of each calendar month. The first Report will cover the period up to the end of the first calendar month following the Consultant’s mobilization. Each Monthly Progress Report will include:

- Brief summary information about any events or circumstances (including ES aspects) which, in the Consultant’s opinion, may create sufficient grounds for any time, claim and/or cost overrun under the Contract and the Consultant’s recommendation of the measures being (or to be) adopted to overcome such events or circumstances and the contractual basis thereof;

- Brief summary information on Contractor’s ESMP development, approval and implementation, the recommendations proposed by the Consultant for improving CESMP developed by the Contractor, the measures taken for solving identified non-conformity/ies on construction site/s and review the effectiveness of correction actions taken by the Contractor to solve non-conformity/ies;

- Comparison in the form of a chart showing the Contractor’s original cumulative cash-flow estimate, in monthly periods, of all payments to which the Contractor/s will be entitled under the Contract and the actual payments certified by the Consultant up to the end of the reporting period. In the event of the cumulative amount of the actual monthly payment certificates being lower than the Contractor’s estimates, the Consultant will accompany the chart with a supporting report and provide:

- Details of any events or circumstances that have caused the discrepancy;

- Assessment of the significance of such events or circumstances, including the Consultant’s opinion on whether these may jeopardise the completion in accordance with the Contract;

- Report on the measures being (or to be) adopted to overcome delays in respect of each event or circumstances and the contractual basis thereof;

- Comparison of the actual percentage completion of delivery compared with the planned for each critical path item identified in the Consultant’s Services Programme Report; where any delivery is behind the Program, the Consultant will comment on the likely consequences and state the remedial action being (or to be) taken;

- Comparison of the actual percentage completion of delivery compared with the planned for each main item of Plant and Materials, if not included in the list of critical path items;

- Projection on Contract’s total final price and the completion date;

- Photographs showing the status of manufacture and of progress on the site;

- List of new quality assurance documents, reports on test results and certificates of materials;

- Safety statistics, as provided by the Contractor, including details of any major incidents and activities (e.g., strikes, riots, demonstrations, media attention, etc.) relating to workers, public, and environment, any type of allegation (sexual exploitation, sexual abuse or sexual harassment), gender and age of the person who experienced the alleged incident should be included in the Report;

- Brief summary information on compliance with applicable environmental and social norms and regulations of Republic of Moldova and the safeguards requirements of the World Bank regarding site organization, black soil storage, consent from local community regarding materials storage, personnel medical control, etc.|).

- List of all notices, consents, approvals, certificates or determinations given or issued by the Consultant within the reported period, including consents to the Contractor’s Performance and the Advance Payment Guarantees;

- Grievance log including list of grievances received, resolved, outstanding issues and redress status, if any; and

- Other information, as may be required by the Client.

The reports have to be concise and contain only critical information related to progress, issues and key events for the month. The Consultant’s Monthly Report should not exceed 20 pages, except for photos which should only be submitted electronically and the above requested copies of the project documents. Only critical correspondence should be provided, as the annexes.

- Change Orders Reports

The Consultant shall note that the Client must seek the no-objection from the Bank prior to the approval of any contract change, modification, claim determination, extension of time and/or approval of additional cost.

The Consultant must therefore prepare Change Orders in sufficient detail, accuracy and clarity to ensure that the Client can comply with the Bank’s procedures for the approval of Contract/s Changes etc.

The Consultant shall immediately advise the Client (in writing) as soon as he becomes aware that a Contract Change order may be required. He shall then prepare and submit within 14 days an analysis of the Change, or in cases where external rates / costs are involved (for example utility works) an estimate of when the analysis will be submitted.

The Consultant must ensure that wherever possible existing or similar rates that exist within the Price Schedule must be used.

The report shall outline the basis for the Consultant’s valuation of the Change including but not limited to the following:

- Whether the activities were unforeseen and under whose risk (Client’s or Contractor’s) the consequences fall in accordance with the Plant Design, Supply and Installation Contract/s;

- Whether the activities fall under Change or not, and if not, then what are the advantages of an amendment of the contract vs bidding the extra scope out;

- Assessment of the time implication with potential prolongation costs and analysis of whether a Change is warranted in a separate bidding procedure;

- The quantity and the value of the varied activities that can be determined using existing rates and prices set out in the Contract;

- The quantity and the Consultant’s estimate of the value of the proposed change, for new plant and/or facilities which can be determined by using similar rates and prices set out in the contract as the basis for valuation;

- The quantity and the Consultant’s estimate of the value of the varied activities that require the development of new rates using external quotations/ cost estimates from specialised sub-contractors (e.g. electrical, communications);

- The Consultant shall provide a detailed breakdown of the rates and prices set out in the contract and identify the price components that the Consultant used or intends to use for the valuation of the proposed change;

- The quantity and the Consultant’s estimate of the value of the varied activities, which can only be determined using the rates and prices not set out in the Contract;

- The Consultant shall provide to the Client an appropriate justification of using rates and prices not set out in the Contract with the evidence of at least three quotes from the market to confirm the reasonableness of the suggested rates.

In case a Change is approved by the Client, the Consultant shall immediately issue the Change Order to the Contractor. He shall adjust the Price Schedule to reflect the changes made by the respective Change Order.

- Report on Completion of works

On completion of the Plant Design, Supply and Installation Contract, upon issue of the Completion Certificate, within 28 calendar days the Consultant shall submit a Completion Report, which shall include but not limited to, copies of the Completion Certificate; verified “as-built” drawings showing all revisions to the design of the plant and facilities; an analysis of the completion cost of the activities; an overview of the actual progress of the activities detailing reasons for delays and/or extensions of time; an overview of Site safety procedures, any problems in this regard and recommendations for improvement; an overview of the Contractor’s working practices and resources; an assessment of the quality of Materials and workmanship, any problems in this regard and recommendations for improvement; details of technical difficulties encountered and how these were overcome; details of administrative difficulties encountered and how these were overcome.

- Claims Reports

The Consultant shall implement the procedures for Claim management as set out in the General Conditions of the Plant Design, Supply and Installation Contract clause 45.

The Consultant shall prepare and maintain a detailed register of all claims and potential claims.

A summary of the status of Claims, Notices of Claim and potential Claims shall be included, updated and reported by the Consultant in each Monthly Progress Report to the Client. The summary must clearly show the current status of each Claim and events that may have occurred in the reporting period, and actions planned by the Consultant.

In the event of the receipt of a Notice of Claim, or Claim from the Contractor/s the Consultant shall immediately notify, and provide a copy of the Contractor’s Notice of Claim to the Client, and record the Notice into the Claims Register.

The Consultant shall conduct a detailed analysis of all claims. The Consultant shall involve and coordinate work with project team members to obtain relevant background information and supporting documentation.

Each Claim must be assessed individually on its relevant merits.

The Consultant shall submit to the Client a preliminary assessment of the validity of a Notice of Claim within 7 days of receipt of the Contractor’s Notice and provide an assessment (including a risk assessment) with regard to the potential impact and outcome of the claim.

The Consultant shall advise what steps can be implemented to mitigate the potential impact of the Claim if it is formally submitted.

The Consultant will propose to the Client solutions that may prevent a Notice of Claim becoming a formal Claim.

The Consultant shall require the Contractor to copy to the Client all details sent to the Consultant with regard to the submission of a Contractor’s Claim/s.

The Consultant shall prepare all necessary letters and transmittals to be issued in respect of claims or potential claims.

- Draft Final Report (Phase II)

Two months prior to the assignment completion, the Consultant shall prepare a first draft final report which shall highlight all major points of interest that arose during the contract implementation. The report will include, amongst others, the summary of the type, quality, quantities and sources of materials used on the contract; Contractor’s plant and personnel; problems encountered and solutions employed; changes made by the Contractor/s in design and specifications and the reasons therefore; a breakdown of Contractor’s performance in terms of respect of the Service Quality Criteria for maintenance services; a breakdown of the final cost of the contract item by item; a summary of Contract changes; a summary of all related accidents happened during the contract execution and other relevant issues.

- Final Report (Phase II)

Upon completion of the Contract, i.e. immediately upon issuing the Final Payment, the Final Report shall be submitted by the Consultant which will take into account all comments, if any, provided by the Client to the draft final report.

- Special Reports

If required and at the request of the Client, within two weeks from any such request, the Consultant shall prepare a Special Report on any major issue raised by the Contract implementation, including (but not limited to) modification of Drawings.

- Report on Defect Liability Period (Phase III)

Upon completion of the 12 months of Defect Liability Period the Consultant shall submit the Report on Defect Liability Period that will include all the identified defects in the design, engineering, materials and workmanship of the Plant supplied or executed works, Consultant’s input in solving them and remedying of the defects by the Contractor.

- Contract Completion Report (Phases I+II+III)

Upon completion of the all Phases (I, II and III) the Consultant shall submit a Report which summarizes the outcomes of the completed phases.

- DELIVERABLES

The Consultant will provide the Client and the Beneficiary with information, designs, data and documentation through submission of periodic reports prepared during the course of its service or specific reports prepared at the request of the Client in relation to the design, construction, completion, testing or commissioning of the Works.

Reporting will include at least the listed above in this ToR, under 4.2. Reporting Requirements and Time Schedule for Deliverables.

For practical reasons it is proposed to allow a delay of 10 (ten) working days in submission of the reports in the Romanian language due to the translation efforts.

All reports issued by the Consultant shall be reviewed and approved by the Client. A period of two weeks shall be allowed for the review and approval.

In addition, the Consultant will:

- Document the work through Working Papers, Issues Papers and Memoranda in Romanian and/or English, as appropriate; and

- Prepare and maintain full and proper records of all meetings and discussions.

# IMPLEMENTATION ARRANGEMENTS

The Client expects to appoint the Consultant in March 2022.

  - Implementation Arrangements

The Plant Design, Supply and Installation Contract is scheduled to be signed in May 2022 and is expected to have a duration of 24 months.

The Services to be provided by the Consultant are expected to last 35 months and will be divided in three (3) Phases.

The first Phase (I) includes 2 months period for bids evaluation activities. The estimated input of Key – experts towards this Phase is 10 man -months.

The second Phase (II) includes 1 month for pre-commencement activities, 20 months of works supervision and closing activities (to be contracted on a time-based principle).

The estimated input of Key – experts towards the Phase (II) is 155 man-months.

The third Phase (III) will cover the supervision and monitoring during Defect Liability Period and will be contracted on the basis of a time-based remuneration. This will be subject to the successful performance of the Consultant in Phases II.

The estimated input of Key – experts towards this Phase (III) is 5 man -months.

The total duration of the services to cover the period till after the issue of the Defects Liability Certificate for the last completed contract is estimated to be 24 months, which will be financed in two (2) stages as follows: Stage 1: for an initial 12 months period financed from the Current Loan Agreement and Stage 2: for a further 12 months period financed by other sources. The Consultant shall prepare a Technical Proposal and a Financial Proposal for both Stage 1 and Stage 2, which will be evaluated on the basis of the aggregate price. The contract to be signed with the Consultant will be for Stage 1 only, and a separate addendum/contract may be signed for Stage 2 provided that the loan is extended or other source of financing is secured and provided the Consultant’s performance under Stage 1 was satisfactory.

The time period may be changed by written agreement between the Parties.

  - Facilities

## FACILITIES TO BE PROVIDED BY THE CLIENT

- Services, facilities and property to be made available to the Consultant by the Client:

- All available information, reports, documents, etc., related to the execution of the Contract shall be made available to the Consultant by the Client. The facilities to be provided to the Consultant are described in the Plant Design, Supply and Installation Contract.

- Professional and support counterpart personnel to be assigned by the Client to the Consultant’s team:

- Counterpart personnel are not required.

The Consultant will supply for its team all necessary computer hardware and software, and printers.

The Beneficiary shall provide, furnish and maintain contract/s offices endowed with necessary office equipment (except for the necessary computer hardware and software, and printers, which shall be supplied by the Consultant, as mentioned above) for use by the Consultant and his staff on the site/s. The contract offices shall be at locations provided by the Beneficiary and acceptable for the Consultant.

In case the Contractor/s will use any proprietary software for design and planning, free use of a license for it would be provided to the Consultant for the duration of the services.

The Contractor/s shall provide Laboratory testing (if necessary), according to the manufacturers’ recommendations.

The Client may assist the Consultant in obtaining any entry/exit visas, etc. However, any related costs shall be borne by the Consultant.

All other costs shall be borne by the Consultant and shall not be reimbursable. The Consultant will provide residential accommodation for their specialists, and local and international transportation, mobile telephones, topographical survey equipment, all necessary local support staff such as secretaries/interpreters, drivers, office assistants, field assistants, as well as any other goods, equipment or services for successful execution of Consultancy Services. The Consultant will also be responsible for all salaries, fees, allowances, insurance, leave pay and taxes for the staff involved in the assignment.

All employees, including the Consultant and the Contractor’s personnel engaged in the activities related to the implementation of the project, must fully respect prescribed measures for occupational protection and also additional measures related to COVID-19 pandemic, all in accordance with the recommendations of the World Health Organisation and authorities of the Republic of Moldova.

The Consultant should also take into account both World Bank’s guidance materials regarding coronavirus pandemic - see https://www.IBRD.com/sustainability-covid.html.

  - Project Data and Documents

All documentation related to the execution of the Contract is and will remain the property of the Client after completion of the assignment. The Consultant shall not publish, use or dispose of this documentation without written consent of the Client.

The Consultant shall provide and maintain orderly working files and a comprehensive, computerized log for correspondence, minutes of meetings and conferences, submittal data, submittal registers, inspection and Monthly Progress Reports, contract documents including amendments, notice to commence, Change Orders and modifications, all in a Project Management Information System (PMIS), as approved by the Client and compatible with the Client’s Management Information System (MIS). The Consultant shall also maintain all detailed deliverable inventory, scheduled dates and actual status. During the course of the activities under the contract, the Consultant shall maintain any and all electronic and printed project documents in good order in its site office. From time to time, the Client may request the Consultant to provide certain documents to interested parties, approved by the Client. Documents of a sensitive nature should be stored separately in the reference library. Prior to completion of the final contract period, the Consultant shall deliver to the Client any and all hard copies project documents, in good order and properly indexed and marked. Additionally, the Consultant shall provide the Client with all electronic files of any and all project documents stored in a media acceptable to the Client including a comprehensive, well-organized electronic index of all those documents. The copyright of all project materials and any software license used for the PMIS shall belong to the Client.

The PMIS will be required to be compatible with standard office, database, project management and CAD software and Windows operating system.

# PROFILE OF THE CONSULTANT

The Consultant is required to provide independent, impartial technical, cost, strategic, management, financial and legal advice, and as such will not be permitted to have a commercial interest in any other contracts or agreements related to the Project.

The Consultant shall prepare technical and financial proposals for the Phases I, II and III and evaluated for the aggregate price.

The Consultant shall employ suitably qualified Experts and other professionals, competent enough to carry out their duties in accordance with responsibilities and/or authorities that are specified in these ToR. The Consultant must be or must include as a subconsultant, a Consultant(s) with a strong track record and demonstrable experience and expertise in the procurement, design, Project Management, construction supervision, participation in Construction of cogeneration plants based on gas engines and high-voltage facilities, Operation of combined heat and power plant, Commissioning/Testing, gained from at least three international projects during the last 15 years.

The Consultant shall retain personnel, with the required minimum qualifications for the assignment, covering the following tasks:

- Project Management

- Project Planning and Scheduling

- Civil Consulting

- Design of cogeneration plants/power plants

- Construction Health & Safety

- Power Engineering

- Electrical Engineering

- Mechanical Engineering

- Telecommunication

- Civil Engineering / Structural Analysis

- Protection through Relays and Automation

- Natural Gas (engineering)

- Monitoring/Supervision of Construction of cogeneration plants and high-voltage facilities

- Environmental & Social Management

- Addressing sexual exploitation, sexual abuse and sexual harassment cases

- Claims Assessment.

The indicative inputs and required qualifications for the project team are presented in the table below. However, the Consultant shall propose the time commitment for each of the key staff in the proposal.

All key personnel shall be fluent in English, and knowledge of the local languages will be considered an asset, and be fully computer literate, word processing, spreadsheet, etc.

Consultants are encouraged to associate with local firms and include in the team local Technical Support Staff.

In the Client view, the Consultant Team shall comprise (but not necessarily be limited to) the following key personnel:

Consultant’s Staffing

| Position | Qualification Requirements |
| --- | --- |
| 1. Key Experts: |  |
| A. Project Manager (Team Leader) | - Minimum M.Sc. degree in electrical / power / mechanical engineering or similar; - Minimum 15 years’ planning, design review, implementation and monitoring of international projects, with regard to Combined Heat and Power Plants based on GICE; - Minimum 10 years’ experience as Project Manager or Team Leader of multi-disciplinary teams in the design and construction under similar projects; - Familiarity with and experience & knowledge of the procurement policy and rules of international financial institutions such as the Word Bank, EBRD and knowledge of Word Bank Plant Design, Supply and Install conditions of contract; - Permanent employee of the Consultant for a period of minimum 3 years within last 7 years. - Fluent knowledge of English. |
| B. Deputy Project Manager | - Minimum M.Sc. degree in power/ electrical / / mechanical engineering or similar; - Minimum 10 years’ planning, design review, implementation and monitoring of similar projects, with regard to Combined Heat and Power Plants or High Voltage installations of at least 110 kV; - Minimum 7 years’ experience as Project Manager of multi-disciplinary teams in the design and construction under similar projects; - Familiarity with and experience & knowledge of the procurement policy and rules of international financial institutions. - Fluent knowledge of English, Romanian and Russian. |
| C. Mechanical Engineer | - Minimum B.Sc. degree in mechanical engineering or similar; - Minimum 15 years’ experience in providing Consulting services for supervision of design, construction and installation works of boilers and GICEs for CHPs. - Experience in the analysis / evaluation of technical solutions / bids within the procurement of goods and works related to the field of activity. |
| D. Electrical Engineer | - Minimum B.Sc. degree in electrical engineering or similar; - Minimum 15 years’ experience in providing Consulting services for planning, design and supervision of construction works of substations / electrical power systems of CHPs or similar (110 kV or higher voltage). - Experience in the analysis / evaluation of technical solutions / bids within the procurement of goods and works related to the field of activity. |
| E. Relay Protection and Automation Engineer | - Minimum B.Sc. degree in electrical / automation engineering or similar; - Minimum 15 years’ experience in supervising the design, construction, adjustment of protection relays and setting up the automation processes, testing and commissioning in power plants / substations or similar. - Experience in the analysis / evaluation of technical solutions / bids within the procurement of goods and works related to the field of activity. |
| F. Environmental, Health & Safety and Social Expert | - Minimum B.Sc. degree in biology, environment or similar. - Minimum 10 years’ relevant work experience in managing occupational health, safety performance including implementation of environmental management plan of infrastructure project of at least one international contract technically similar in complexity. The specific experience process, according to Environmental and Social standards of World Bank and/or other IFI’s as well as according to Local laws would be an advantage. - Experience in the analysis / evaluation of technical solutions / bids within the procurement of goods and works related to the field of activity. |
| G. Procurement Expert | - Minimum bachelor's degree in fields related to the procurement process (e.g. law, finance, economics, etc.). - Minimum 15 years of experience in the field of procurement of goods, services and works, in accordance with the regulations and guidelines of the International Financial Institutions, including the World Bank, and in compliance with the necessary procurement procedures. - Experience or participation in the evaluation of Bids for works contracts in accordance with the regulations of the World Bank and / or International Financial Institutions, including the preparation of Bid Evaluation Reports. |
| 2. Non-Key Experts |  |
| H. Mechanical Engineer | - Minimum B.Sc. degree in electromechanical or mechanical engineering or similar; - Minimum 10 years of experience in reviewing the design, supervision of construction and installation works of boilers, engines and/or turbines in power plants. |
| I. Civil Engineer | - Minimum B.Sc. degree in civil engineering or similar; - Minimum 15 years’ experience in supervising the design and construction of Power Plant buildings (preferably CHP) / GICE halls or similar, including foundations (specifically for heavy engines). - Experience in the analysis / evaluation of technical solutions / bids within the procurement of goods and works related to the field of activity. |
| J. Electrical Engineer | - Minimum B.Sc. degree in electrical engineering or similar; - Minimum 8 years of experience in construction and providing services for supervision of installation works of electrical equipment for power plants. - Minimum 5 years’ experience in the field in the Republic of Moldova or CIS countries. |
| K. Gaz Engineer | - Minimum B.Sc. degree in gas supply system engineering or similar. - Minimum 10 years of experience in maintenance, repair, installation and testing of the gas equipment. - Experience in integrating gas engines in different installations in thermal power plants shall be an advantage. - Minimum 5 years of activity in the field in the Republic of Moldova or the CIS countries. |
| L. SCADA / Telecommunication Engineer | - Minimum B.Sc. degree in relevant power system/telecommunication/technical discipline; - Minimum 10 years of experience in construction of Power Plants or Substations; - Experience in preparing /reviewing the design/ drawing of protection and control system/supervision of installation, testing & commissioning of protection and control systems. |
| M. Geologist / Geotechnical Engineer | - Minimum B.Sc. degree in relevant technical discipline; - Minimum 10 years of experience in geotechnical engineering and soil mechanics. He should be familiar with local condition, slope and embankment construction, laboratory and in-situ testing and foundation investigations; - experience in construction of power plants (preferably based on GICEs). - Experience in the field in the Republic of Moldova or the CIS countries. |
| N. Commissioning Expert | - Minimum Master degree in relevant power system/technical discipline; - Minimum 10 years of experience supervision of construction and commissioning of Power Plants (preferably based on GICEs); - Experience in reviewing the design, supervision of installation, testing & commissioning of Power Plants (preferably based on GICEs). |
| O. Financial/Economic Expert | - Minimum B.Sc. degree in economics, finance, accounting or similar. - Experience of at least 10 years in the field of finance, economy or similar. - Experience in reviewing / analyzing / evaluating the financial capacity of bidders in accordance with the requirements of international financial organizations, including the World Bank. - Experience in evaluating financial bids in public or private procurement. |